Debug for RT600

We use jtag's swd interface to debug the program, here we need to link the application to ram.

- We use the MCUXpresso IDE to debug the rt600. First, we need to link the application to ram, otherwise debug fails.

- Connect jlink to mainboard

There are two ways.

2.1  Use RT685 EVK board,  debug external DUT.

Remove EVK board JP18 and JP19 ,  JP18 pin2 connects  external DUT‘s SWCLK pin.  JP19 pin2 connects  external DUT‘s SWDIO pin. At the end, connect them GND together.

  - Use jlink, There may be a voltage difference problem.

The J-link boxes must be licensed，Because our IO level of the main board is 1.8v, the IO level of the J-link box needs to be the same as that of the main board. Measure the PIN1 voltage of J-link before connecting the motherboard. If it is 3.3v, this J-link is not available. If it is no voltage or 1.8v, it can be used, At the same time, need connection SYS_1.8v for JLINK identification.

JLINK need V9 and V10.